| My Oxygen Sensor Change 2001-2005 Aztek 3.4L V6 « on: December 15, 2007, 04:36:00 pm » |  |
| --- | --- |

Just got done changing my before cat oxygen sensor and the job was not bad at all....Just like my spark plug/wire change, the key was to move the coilpack.
Two 10mm bolts on the front of the coilpack and two 13mm nuts on the back. This gives you room to work.
1. Take off the bolts/nuts for the coilpack and move it up and to the driver's side. You can see/feel the O2 sensor in the exhaust manifold.
2. The O2 sensor harness is plugged into the coilpack and is attached with a plastic nipple to the coilpack frame. I cut the nipple off as the new sensor had one. Unplug the harness. 
3. I used an O2 sensor socket, which is a socket with one side cut out to let the wire out. Once loose, the sensor can be taken out by hand. 
4. The sensor already had anti-seize on the threads. If you want to put some on, do not get any anti-seize on the sensor itself...It will die a quick and expensive death. Put the new one in by hand till snug, the tighten with the O2 socket.
5. Plug the sensor into the harness, and snap the plastic nipple back into the coilpack frame.
6. Set the coilpack back on the studs, and tighten down the nuts/bolts.

Took me about 40 minutes. No rolling the engine, and no bloodshed!

Would have been better if I would have had the sensor when I did my plugs/wires!

Thanks to the Aztek Forum for this tip.